Programming Mitsubishi Keyless Remote Instructions (on my '08 Evo X; yours may be slightly different):

PROGRAMMING:
(Note: Entering new remote transmitter codes will erase all old codes so all remote transmitters must be programmed together.)

- Ensure ignition switched OFF.

- Locate and access your ODBII connector (aka DLC terminal) under dash (driver side by left knee).

- Jump DLC terminal's PIN 1 and 4 (which is ground). BTW, if you feel the ODBII connector under the dash, you can tell that it's a "D" shaped connector with the widest side towards the driver and not the firewall. The connector consists of 2 rows of contacts with 8 pin connectors on the wide side and 7 on the narrow side. PIN 1 is on the wide side (which is closest to the driver) and is the most left PIN (closest to the driver side door).
(Note: To do this, I used a piece of wire that's used as ties from a grocery store, stripped the ends and made a loop on both ends by wrapping it around a toothpick, then I got out my ODBII cable from my (Correction: COBB AccessPORT cable and NOT my ODBII meter cable because PIN 1 was missing from that. Cable only; do NOT have your AccessPORT connected) and shorted pins 1 and 4 by placing the loops of my newly made wire on both of those pins, and lastly you can connect the ODBII cable onto the vehicle to short the two pins)

- Within 10 seconds: Press hazard switch six times.

- My doors did NOT lock and unlock as many other Mitsubishi instructions have stated.

- Immediately within 10 seconds: Press remote transmitter LOCK button a couple times until your door locks respond (other instructions say 2 or 3 times but when it registers and your door locks respond, you are good).

- If registering additional remote transmitters, within 1 minute (I did it within 10 seconds): Repeat the previous step and proceed to each additional new remote transmitter until all are done.

Note: A maximum of 4 remote transmitters can be programmed. 

Note: To exit programming mode: Remove ODBII connector or remove jump lead.

- Test each remote again to verify that each has been programmed correctly.
________________________________________________________________

Yesterday, my battery was low on one of my keyless remotes as noted on my dash when I tried to start the car. Since you get 2 from the dealer, I went into the house to swap keyless remotes and to my surprise after not using the second remote for almost a year, it failed to work. Hence the reason why I've put this DIY tutorial together for you to program your own keyless remote.

This is so easy to do that it begs the question, can I go to someone else's EVO with my keyless remote, and armed with a piece of wire program my keyless remote to start their car, LOL......hmmm.


--GeeDubz 

